THE GRAIL COLLECTOR

Roadmap Rev I — Completion Status (updated, Rev 4.2)

Item-by-item status of the Design and Simulation Roadmap (Rev I, corrected) after the CFD phase and Stages 3, 5, 6 and 7 (ANN, NSGA-II, system model, annual analysis). The roadmap itself is not rewritten; this document records what was done, how, and what remains.

| Item | Value |
| --- | --- |
| Roadmap version | Rev I (corrected), 64 pages |
| Phases closed | CFD (Sections 4–8, 12, 15); ANN (10); system model and annual analysis (11, via Python, GNU Octave, Scilab Xcos and MATLAB/Simulink, all agreeing) |
| Not done | Section 9 topology optimisation (replaced by NSGA-II parameter optimisation on the ANN surrogate) |
| Final dataset | GRAIL_CFD_dataset_FINAL_rev4.2.csv — 300 cases, audited (Rev 4.1 data, three columns renamed) |
| Main deviation | COMSOL 6.2 → OpenFOAM v1912 (3-D) + GRAIL-CHT reduced-order conjugate model |
| Experimental validation | None of the present collector. V5 COMPLETE (method/component level): 16 papers screened; method checks against Del Col 2013 (roll-bond: a1, a2 within uncertainty, η0 +6–8 %) and Beikircher 2015 (TIM front loss within measured range; VIP rear loss optimistic) |

# Contents

# 1. Section-by-section status

| Roadmap section | Status | What was done / why not |
| --- | --- | --- |
| 1–3 Thesis, gaps, prior art | REFERENCE | Background; not a work item |
| 4 Design parameter set | USED | Groups A, B, E sampled: G_T, T_in, T_amb, v_wind, ṁ, bridge, G, λ_G, f_interdig |
| 5 Architecture, BOM, properties | USED | Material database recorded; V6 supplier datasheets not obtained |
| 6 Fusion 360 modelling | DONE | Grail_Collector_2.step, 129 solids; Gate A 12/12 invariants PASS |
| 7 Fusion → COMSOL | REPLACED | Direct NURBS evaluation and structured hex meshing for OpenFOAM |
| 8.1–8.6 CFD methodology | DONE (deviation) | OpenFOAM simpleFoam + chtMultiRegionSimpleFoam (frozenFlow); GRAIL-CHT for the campaign |
| 8.7 Validation | PARTIAL | Code-to-code (3-D conjugate) and code-to-correlation (H1 Nu, HWB); Gunjo 2017 rejected; Sun et al. not obtained |
| 8.8 Campaign | DONE | 300 LHS cases, 0 rejected (Rev 4.1); log fields as specified |
| 9 Topology optimisation | NOT DONE | Replaced by NSGA-II parameter optimisation on the ANN surrogate (Stage 5); 8 optima re-run in GRAIL-CHT |
| 10 ANN / hybrid ANN | DONE | Stage 3: 10-seed MLP ensemble, test R² 0.9994–0.9998 (η), GPR benchmark, SHAP |
| 11 MATLAB/Simulink | DONE (deviation) | Stage 6/7 system model in Python, GNU Octave (PASS vs Python) and Scilab Xcos block diagram (0.04 % vs Octave); Simulink V20 run in MATLAB Online by the user: all 4 cases PASS vs Xcos (solar heat within 0.05 %) |
| 12 Results and figures | MOSTLY DONE | See Section 3 |
| 13 Patent strategy | NOT ACTIONED | Engineering cross-check only; V1, V10 open |
| 14 Schedule and risk | REFERENCE | — |
| 15 Rev H corrections | DONE | Converging geometry (C1) used throughout; C7, C8 open (manufacturing dimensions) |

# 2. Verification register V1–V10

| # | Item | Status | Evidence / note |
| --- | --- | --- | --- |
| V1 | Prior-art null results | FIRST PASS DONE | Web first pass (20_Patent_prior_art): no document found combining roll-bond + converging channels (+ TIM/PCM/VIP); inventive step weak–moderate; professional search on Espacenet/Patentscope/InPASS pending, only if filing |
| V2 | COMSOL licence | NOT APPLICABLE | COMSOL not used |
| V3 | MATLAB licence | DONE | Section 11 in GNU Octave, Scilab Xcos and MATLAB/Simulink; the user ran GRAIL_simulink_complete.m in MATLAB Online and reported the result as matching (PASS vs Xcos); V20 printed output archived in 17_Simulink_MATLAB_model/V20_run_record.md (all 4 cases PASS) |
| V4 | COMSOL feature names | NOT APPLICABLE | OpenFOAM v1912; every solver option verified in the installed binary |
| V5 | Baseline validation < 8 % | DONE (partial pass, method level) | 16 papers screened (folder 11). Del Col 2013 method check (folder 19): heat loss within test uncertainty, η0 over-predicted 6–8 %. Beikircher 2015 component check (folder 21): TIM front loss within measured range; catalogue VIP conductivity optimistic (a1 2.1 → 2.3–2.5 W/m²K). η0 within 8 % (+6.0 / +7.8 %); curve within 8 % up to Tm* 0.02 m²K/W. No experimental validation of the present collector is claimed. Closed by the user; record: 11_Validation_V5/V5_FINAL_closure.md |
| V6 | Supplier datasheets | OPEN | Literature properties used |
| V7 | Counter-current effect on R4, matched mean T | DONE | GRAIL-CHT at matched plate mean 320/330/340 K (29_v7): Δη = −0.001 points, ΔR4 = +0.04 %, plate std +8 to +11 % for alternating; alternating needs ~15 K cooler inlet for the same plate temperature |
| V8 | η0, a1, a2 fit | DONE | Stage 6 ISO 9806 fit on 22 GRAIL-CHT runs: alternating η0 0.597, a1 2.00 W/m²K; parallel η0 0.637, a1 2.10 W/m²K; held-out errors ≤ 0.24 points |
| V9 | TO geometry re-simulated | NOT APPLICABLE | No TO |
| V10 | Patent-agent review | OPEN | — |

# 3. Figures (roadmap 12.1)

| # | Figure | Status | Delivered as |
| --- | --- | --- | --- |
| F1 | Grid independence | DONE | F01 (3-D axial), F02 (cross-section Nu), F04 (ROM axial) |
| F2 | Validation curve vs Gunjo/Sun | NOT POSSIBLE | Reference rejected; F10 Nu envelope and F05 ROM-vs-3-D instead |
| F3 | Efficiency curves conventional vs GRAIL | PARTIAL | GRAIL curves done (F18, Stage 6); no conventional collector modelled |
| F4 | Geometry family incl. TO | PARTIAL | CAD report figures; no TO |
| F5 / F8 | Temperature contours; counter-current side by side | DONE | F09 3-D rear-surface maps, same scale |
| F6 | Velocity field | DONE | CAD/CFD report figures S3, S6; videos |
| F7 | Plate/fluid T along flow | DONE | F08 |
| F9 | Temperature histogram | DONE | F09b |
| F10 | R4 vs bridge width | DONE | F10b (T_R4 − T_mean vs bridge) |
| F11 | Loss breakdown | DONE | F11 |
| F12 | Correlation heatmap | DONE | F12 |
| F13 / F14 | Grading result and penalty | DONE | F13_F14 |

# 4. The three headline numbers (roadmap 12.3)

| # | Roadmap sentence | Answer from the closed CFD phase |
| --- | --- | --- |
| 1 | Counter-current reduced R4 by __ % at unchanged mean T, raising η by __ points | ANSWERED (V7): at matched mean plate temperature counter-current changes R4 by +0.04 % (not a reduction) and η by −0.001 points (no change); plate std is 8–11 % higher. At unchanged inputs alternating runs +13.9 K hotter and −7.3 % less efficient because it needs a ~15 K cooler inlet to reach the same plate temperature |
| 2 | TO absorber delivered __ % more | NOT APPLICABLE — no TO |
| 3 | Annual kWh/m²/year | DONE (Stage 7, Berhampur TMY): collector yield at 40 °C inlet 1,030 (parallel) / 965 (alternating) kWh/m²/yr; POA 1,976 kWh/m²/yr |

Consequence for Filing 1

The roadmap (15.8) warns that Filing 1 is a claim about a flatter temperature field. The corrected CFD shows a flatter field only at high flow; at matched mean plate temperature (V7) the arrangements have the same efficiency and alternating is slightly less uniform at the design flow. Claim language must match this evidence.

# 5. Roadmap 15.8 corrected sequence

| Step | Status |
| --- | --- |
| 1 Re-export STEP with suppressions | DONE (as direct geometry evaluation) |
| 2 Isothermal mass check within 0.1 % | DONE — mass imbalance ~1e-9 |
| 3 Reynolds 23.9–198.8 | DONE — Rev 4.1 Re_in 21.8–151.3 (correct non-circular definition; low end below range because ṁ bound 1.0 g/s) |
| 4 Validation gate 8.7 | PASSED at method level (V5); no test of the present collector |
| 5 Variants; counter-current at matched mean T | DONE — see V7 (29_v7) |
| 6 w_bridge 20–60 mm with r_fillet 0.5–2.0 mm | PARTIAL — 20–37 mm (geometric hard limit at 40 mm pitch); fillet not swept. DESIGN_REVIEW_REQUIRED |

# 6. Remaining open items

- V5: closed at method/component level (Del Col 2013, Beikircher 2015). A prototype test would be needed for experimental validation of GRAIL itself. DESIGN_REVIEW_REQUIRED: use a measured in-collector VIP conductivity (0.010–0.018 W/m·K).

- V1 professional search and V10 patent-agent review, only if a filing is planned (first-pass search in 20_Patent_prior_art).

- V6: supplier datasheets (literature properties used).

- C7, C8: frame wall thickness and manifold dimensions — design inputs from the user.

- Bridge range above 37 mm (needs a different pitch) and the fillet sweep.

- Design decisions recorded: 2 modules is the system design (4 modules stagnate above 420 K — overheating protection needed for larger arrays); the NSGA-II optimum at the flow bound with G → 1 means the converging channel is argued for plate uniformity, not efficiency.

- Deferred by choice: PCM latent heat, radiation / optical 3-D model, full 3-D campaign.